CPE 166 Advance Logic Design

Harpreet Sidhu

Lab Section #3

Lab 3 Report

David Quintanilla

Table of Contents

- Introduction	….……………………………………………………………………. 3

- Part 1 : (7,4) Hamming Code Generator …………………………………………….. 4- 11

  - Step 1 Parity Function	  ………………………………………………………. 4

  - Step 2  Even Parity Bit of 2-bit Input Data …………………………………… 5-6

  - Step 3 (7,4) Hamming Code Generator ……………………………………7 -11

- Part 2 : Pseudorandom Number Generator ……………………………………….. 12 - 14

  - Step 1 LFSR Design Review

  - Step 2 LFSR Testbench Review for Fig 2. Circuit

  - Step 3 LFSR Design and Testbench Simulation of Fig 1. Circuit ………… 12 - 14

- Part 3 : Algorithmic State Machine (ASM) Charts	………………………………15 - 20

  - Step 3 Final Project Work…………………………………………………..15 - 20

- Part 4: Stopwatch Design …………………………………………………………. 21 - 36

  - Step 1 clkdiv block …………………………………………………………21 - 23

  - Step 2 FSM block  ………………………………………………………….23 - 27

  - Step 3 Watch block …………………………………………………………27 - 32

  - Step 4 Final stopwatch block ……………………………………………….33 - 36

- Conclusion ………………………………………………………………………….. 37

Introduction

Lab 3 introduced a wide variety of topics including the introduction to Hamming Code, Linear Feedback Shift Registers and Algorithmic State Machines (ASM). In Part 1, I learned a basic understanding of Hamming Code. Consisting of bits and Parity bits, they key of Hamming code,  is the use of extra parity bits to allow the correction of a single bit error. Furthermore learning that an even parity but is the bit added to the data block that ensures that the total number of buts in the message is even for error detection. Out of all the parts in Lab 3, Hamming Code was the one that stuck with me the most. Part 2 introduced the Linear Feedback Shift Register, which is a shift register whose input but is the output of a linear logic function of two or more of its previous states. The LFSR was something I was very familiar with so Part 2 was fairly simple to follow along.

Moving along to Part 3, Algorithmic State Machines (ASM) were introduced  and were to be written in VHDL. ASM’s consisted of a State box, Optional decision box and a Optional conditional output box. Part 3 was quite challenging for me due to my lack of familiarity with VHDL, however the concepts of ASM i understood after a couple days practice. ASM were also a key for Part 4. The final part, Part 4 asked to design a Stop watch following a set of guidelines giving in the procedures. Part 4 was by far the hardest part of the lab and required several attempts to finally have a complete running waveform.

Part 1 : Hamming Code Generator

Step 1 Parity Function

library ieee;

use ieee.std_logic_1164.all;

package MY_PACK is

function PARITY (D : std_logic_vector) -- declaration of function

return std_logic;

end MY_PACK;

package body MY_PACK is

function PARITY (D : std_logic_vector) -- implementation of function inside the package body

return std_logic is

variable TMP : std_logic;

begin

TMP := D(0);

for J in 1 to D'high loop

TMP := TMP xor D(J);

end loop; -- works for any size of D

return TMP;

end PARITY; -- even parity

end MY_PACK;

Step 2 Even Parity Bit of 3-bit input Data (Verilog)

library ieee;

use ieee.std_logic_1164.all;

use work.MY_PACK.all;

entity PAR is

port( db: in std_logic_vector(2 downto 0);

pb: out std_logic);

end PAR;

architecture ARCH of PAR is

begin

pb <= PARITY(db);

end ARCH;

Step 2 Even Parity Bit of 3-bit input Data (Test Bench)

library IEEE;

use IEEE.std_logic_1164.all;

entity PAR_tb is

end PAR_tb;

architecture Behavioral of PAR_tb is

component PAR

port( db: in std_logic_vector (2 downto 0) ;

pb: out std_logic );

end component;

signal db: std_logic_vector (2 downto 0) ;

signal pb: std_logic;

begin

uut: PAR port map(db => db, pb => pb);

process begin

db<="000";

wait for 200 ns;

db<="001";

wait for 200 ns;

db<="010";

wait for 200 ns;

db<="011";

wait for 200 ns;

db<="100";

wait for 200 ns;

db<="101";

wait for 200 ns;

db<="110";

wait for 200 ns;

db<="111";

wait for 200 ns;

wait;

end process;

end Behavioral;

Wave Form (PAR)

Step 3 :  (7, 4) Hamming Code Generator (Verilog)

library ieee;

use ieee.std_logic_1164.all;

use ieee.std_logic_arith.all;

use ieee.std_logic_unsigned.all;

use work.MY_PACK.all;

entity hamming is

port( d3: in std_logic;

d5: in std_logic;

d6: in std_logic;

d7: in std_logic;

dout: out std_logic_vector(7 downto 1));

end hamming;

architecture behavioral of hamming is

begin

dout <=  ((D7) & (D6) &(D5) & (PARITY(D7&D6&D5)) & (D3) & (PARITY(D7&D6&D3)) & (PARITY(D7&D5&D3)));

end behavioral;

Test Bench

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity hamming_tb is

end hamming_tb;

architecture Behavioral of hamming_tb is

component hamming

port( d3: in std_logic;

d5: in std_logic;

d6: in std_logic;

d7: in std_logic;

dout: out std_logic_vector(7 downto 1));

end component;

signal d3:  std_logic;

signal d5:  std_logic;

signal d6:  std_logic;

signal d7:  std_logic;

signal dout:  std_logic_vector(7 downto 1);

begin

uut: hamming port map( d3 => d3 , d5 => d5 , d6 => d6,  d7 => d7 , dout => dout);

process begin

d7<=’0’;d6<=’0’; d5<=’0’; d3<=’0’;

wait for 200 ns;

d7<=’0’;d6<=’0’; d5<=’0’; d3<=’1’;

wait for 200 ns;

d7<=’0’;d6<=’0’; d5<=’1’; d3<=’0’;

wait for 200 ns;

d7<=’0’;d6<=’0’; d5<=’1’; d3<=’1’;

wait for 200 ns;

d7<=’0’;d6<=’1’; d5<=’0’; d3<=’0’;

wait for 200 ns;

d7<=’0’;d6<=’1’; d5<=’0’; d3<=’1’;

wait for 200 ns;

d7<=’0’;d6<=’1’; d5<=’1’; d3<=’0’;

wait for 200 ns;

d7<=’0’;d6<=’1’; d5<=’1’; d3<=’1’;

wait for 200 ns;

d7<=’1’;d6<=’0’; d5<=’0’; d3<=’0’;

wait for 200 ns;

d7<=’1’;d6<=’0’; d5<=’0’; d3<=’1’;

wait for 200 ns;

d7<=’1’;d6<=’0’; d5<=’1’; d3<=’0’;

wait for 200 ns;

d7<=’1’;d6<=’0’; d5<=’1’; d3<=’1’;

wait for 200 ns;

d7<=’1’;d6<=’1’; d5<=’0’; d3<=’0’;

wait for 200 ns;

d7<=’1’;d6<=’1’; d5<=’0’; d3<=’1’;

wait for 200 ns;

d7<=’1’;d6<=’1’; d5<=’1’; d3<=’0’;

wait for 200 ns;

d7<=’1’;d6<=’1’; d5<=’1’; d3<=’1’;

end process;

end Behavioral;

Wave Form

Part 2 Pseudorandom Number Generator

Step 1, 2, 3 LFSR Design (Verilog)

Library ieee;

Use ieee.std_logic_1164.all;

Entity LSFR is

Port ( reset, clk: in std_logic;

Q: out std_logic_vector (4 downto 0) );

End LSFR;

Architecture beh of LSFR is

Signal m: std_logic_vector (4 downto 0);

Begin

Process(reset, clk)

Begin

If ( reset = '1') then

m <= ( 0=> '1', others =>'0'); --- value of “0001”

elsif (rising_edge(clk) ) then

m(4 downto 1) <= m(3 downto 0)

m(0) <= m(2) xor m(3);

end if;

end process;

Q <= m;

end beh;

Step 1, 2, 3 LFSR Design (Test Bench)

Library ieee;

Use ieee.std_logic_1164.all;

Entity LSFR_tb is

End LSFR_tb;

Architecture tb of LSFR_tb is

signal clk, reset: std_logic;

signal Q: std_logic_vector (4 downto 0);

component LSFR

Port ( reset, clk: in std_logic;

Q: out std_logic_vector (4 downto 0) );

End component;

Begin

uut: LSFR port map(reset, clk, Q);

Process Begin

Clk <= '0';

Wait for 5 ns;

Clk <= '1';

Wait for 5 ns;

End process;

Process Begin

Reset <= '1';

Wait for 2 ns;

Reset <= '0';

Wait for 200ns;

Wait;

End process;

End tb;

Wave Form

Part 3: Algorithmic State Machine (ASM) Charts

- Steps 1 & 2 are review of ASM charts and code that we learned in class

Step 3. Final Project Work

Library ieee;

Use ieee.std_logic_1164.all;

entity ASM is

port(

reset, clk, x:   in std_logic;

y1: out std_logic;

y0, z: out std_logic_vector (1 downto 0);

ckcs, ckns: out std_logic_vector (1 downto 0)

);

end ASM;

architecture Behavioral of ASM is

constant S0: std_logic_vector(1 downto 0) := "00";

constant S1: std_logic_vector(1 downto 0) := "01";

constant S2: std_logic_vector(1 downto 0) := "10";

constant S3: std_logic_vector(1 downto 0) := "11";

signal cs, ns: std_logic_vector(1 downto 0);

begin

ckns <= ns;

ckcs <= cs;

process(reset, clk) begin

if(reset = '1') then

cs <= S0;

elsif(rising_edge(clk)) then

cs <= ns;

end if;

end process;

process(cs, x) begin

case(cs) is

when S0 =>

if(x = '1') then

ns <= S1;

else

ns <= S0;

end if;

when S1 =>

if(x ='1') then

ns <= S1;

else

ns <= S2;

end if;

when S2 =>

if(x ='1') then

ns <= S1;

else

ns <= S3;

end if;

when S3 =>

ns <= S0;

when others =>

ns <= S0;

end case;

end process;

y1 <= '1' when (cs = S1) else '0';

y0(0) <= '1' when (cs = S2) else '0';

y0(1) <= '1' when (cs = S3) else '0';

z(0) <= '1' when (cs = S1) and (x = '1') else '0';

z(1) <= '1' when (cs = S2) and (x = '1') else '0';

end Behavioral;

Step 3. Final Project Work (Test Bench)

library IEEE;

use IEEE.std_logic_1164.ALL;

entity ASM_tb is

end ASM_tb;

architecture Behavioral of ASM_tb is

component ASM

port(

reset, clk, x: in std_logic;

y1: out std_logic;

y0, z: out std_logic_vector (1 downto 0);

ckcs, ckns: out std_logic_vector (1 downto 0)

);

end component;

signal reset, clk, x, y1: std_logic;

signal y0, z: std_logic_vector (1 downto 0);

signal ckcs, ckns: std_logic_vector (1 downto 0);

begin

uut: ASM port map(reset => reset, clk => clk, x => x, y1 => y1, y0 => y0, z => z, ckcs => ckcs, ckns => ckns);

process begin

clk <= '0';

wait for 5 ns;

clk <= '1';

wait for 5 ns;

end process;

x <= '1', '0' after 10 ns, '1' after 40 ns, '0' after 60 ns, '1' after 80ns, '0' after 120ns,  '1' after 160 ns, '0' after 200 ns, '1' after 300 ns, '0' after 350 ns;

process begin

reset <= '1';

wait for 2 ns;

reset <= '0';

wait for 200 ns;

wait;

end process;

end Behavioral;

Wave Form

Part 4 : Stopwatch Design

Step 1: clkdiv block (Verilog)

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity clkdiv is

port(clkin: in std_logic;

clkout: out std_logic := '0' );

end clkdiv;

architecture Behavioral of clkdiv is

signal cnt : integer range 0 to 10 := 0;

begin

process(clkin) begin

if(rising_edge(clkin)) then

if(cnt = 10) then

cnt <= 0;

clkout <= '1';

elsif (cnt < 3) then

cnt <= cnt + 1;

clkout <= '1';

else

cnt <= cnt + 1;

clkout <= '0';

end if;

end if;

end process;

end Behavioral;

Step 1 clkdiv (Test Bench)

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity clkdiv_tb is

--  Port ( );

end clkdiv_tb;

architecture Behavioral of clkdiv_tb is

component clkdiv

port ( clkin : in std_logic;

clkout : out std_logic );

end component;

signal clkin, clkout : std_logic := '0';

begin

DUT : clkdiv

port map(clkin=>clkin,clkout=>clkout);

process begin

wait for 10 ns;

clkin <= not clkin;

end process;

end Behavioral;

WaveForm

Step 2 : FSM Block

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity FSM is

Port (clk, start, stop, reset: in std_logic;

en: out std_logic);

end FSM;

architecture Behavioral of FSM is

constant s0: std_logic_vector(1 downto 0) := "00";

constant s1: std_logic_vector(1 downto 0) := "01";

signal cs, ns: std_logic_vector(1 downto 0);

begin

process(reset, clk)

begin

if(reset = '1') then

cs <= S0;

elsif (rising_edge(clk)) then

cs <= ns;

end if;

end process;

process(cs, start, stop)

begin

case(cs) is

when S0 => if (start='1') then

ns <= S1;

else

ns <= S0;

end if;

when S1 => if (stop='1') then

ns <= S0;

else

ns <= S1;

end if;

when others => ns <= S0;

end case;

end process;

en <= '1' when (cs = S1 and Start = '1') else '0';

end Behavioral;

Step 2 FSM Block (Test Bench)

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity fsm_tb is

end fsm_tb;

architecture Behavioral of fsm_tb is

component fsm

port (clk, start, stop, reset: in std_logic;

en: out std_logic);

end component;

signal clk, start, stop, reset, en: std_logic;

begin

DUT: fsm port map(clk, start, stop, reset, en);

process

begin

clk <= '0';

wait for 1 ns;

clk <= '1';

wait for 1 ns;

end process;

process

begin

start <= '0';

stop <= '0';

reset <= '1';

wait for 10 ns;

reset <= '0';

stop <= '1';

wait for 20 ns;

stop <= '0';

start <= '1';

wait for 40 ns;

start <= '0';

stop <= '1';

wait for 100 ns;

wait;

end process;

end Behavioral;

Step 2 Wave Form

Step 3 : Watch (Verilog)

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

entity watch is

Port (en, clk, reset: in std_logic;

y0, y1, y2: out std_logic_vector(3 downto 0));

end watch;

architecture Behavioral of watch is

signal y0_reg: std_logic_vector(3 downto 0);

signal y1_reg: std_logic_vector(3 downto 0);

signal y2_reg: std_logic_vector(3 downto 0);

begin

process(clk, reset)

begin

if(reset = '1') then

y0_reg <= (others=>'0');

y1_reg <= (others=>'0');

y2_reg <= (others=>'0');

elsif rising_edge(clk) then

if(y0_reg = 9) then

y0_reg <= (others=>'0');

elsif(en = '1') then

y0_reg <= y0_reg + 1;

end if;

if(y0_reg = 9) then

if(y1_reg = 9) then

y1_reg <= (others=>'0');

else

y1_reg <= y1_reg + 1;

end if;

end if;

if(y0_reg = 9 and y1_reg = 9) then

if(y2_reg = 9) then

y2_reg <= (others=>'0');

else

y2_reg <= y2_reg + 1;

end if;

end if;

end if;

end process;

y0 <= y0_reg when (en = '1') or (reset = '1');

y1 <= y1_reg when (en = '1') or (reset = '1');

y2 <= y2_reg when (en = '1') or (reset = '1');

end Behavioral;

Step 3: Watch (Test Bench)

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity watch_tb is

end watch_tb;

architecture Behavioral of watch_tb is

component watch

port (en, clk, reset: in std_logic;

y0, y1, y2: out std_logic_vector(3 downto 0));

end component;

signal en, clk, reset: std_logic;

signal y0, y1, y2: std_logic_vector(3 downto 0);

begin

DUT: watch port map(en, clk, reset, y0, y1, y2);

process

begin

clk <= '0';

wait for 1 ns;

clk <= '1';

wait for 1 ns;

end process;

process

begin

en <= '0';

reset <= '1';

wait for 20 ns;

en <= '1';

wait for 20 ns;

reset <= '1';

wait for 20 ns;

reset <= '0';

wait for 20 ns;

en <= '0';

wait for 20 ns;

en <= '1';

wait for 20 ns;

en <= '0';

wait;

end process;

end Behavioral;

Step 3 ( Wave Form)

Step 4 Final Stopwatch (Verilog)

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

entity stopwatch is

Port (start, stop, clk, reset: in std_logic;

y0, y1, y2: out std_logic_vector(3 downto 0));

end stopwatch;

architecture Behavioral of stopwatch is

signal en, clk2: std_logic;

component clkdiv

port (clkin: in std_logic;

clkout: out std_logic);

end component;

component fsm

port (clk, start, stop, reset: in std_logic;

en: out std_logic);

end component;

component watch

port (en, clk, reset: in std_logic;

y0, y1, y2: out std_logic_vector(3 downto 0));

end component;

begin

g1: clkdiv port map(clkin => clk, clkout => clk2);

g2: fsm port map(clk => clk2, start => start, stop => stop, reset => reset, en => en);

g3: watch port map(en => en, clk => clk2, reset => reset, y0 => y0, y1 => y1, y2 => y2);

end Behavioral;

Step 4 Final Stopwatch (Test Bench)

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity stopwatch_tb is

end stopwatch_tb;

architecture Behavioral of stopwatch_tb is

component stopwatch

port (start, stop, clk, reset: in std_logic;

y0, y1, y2: out std_logic_vector(3 downto 0));

end component;

signal start, stop, clk, reset: std_logic;

signal y0, y1, y2: std_logic_vector(3 downto 0);

begin

DUT: stopwatch port map(start, stop, clk, reset, y0, y1, y2);

process

begin

clk <= '0';

wait for 1 ns;

clk <= '1';

wait for 1 ns;

end process;

process

begin

start <= '0';

stop <= '0';

reset <= '1';

wait for 20 ns;

reset <= '0';

stop <= '1';

wait for 20 ns;

stop <= '0';

start <= '1';

wait for 20 ns;

reset <= '1';

wait for 20 ns;

reset <= '0';

wait;

end process;

end Behavioral

Step 4 Final Stop watch (Wave Form)

Conclusion

Lab 3 had its on set of challenges that were both familiar and new. One of the biggest lessons learned in this lab was understanding VHDL in a deeper context and the concept of Hamming Code. Some parts of this lab overlap with previous labs such as the clk divider and FSM. A key concept of lab 3 that I took away was “port mapping”. Although this is not a new concept to me, I really understood the concept of porting in lab 3.