CPE 166 Advance Logic Design

Harpreet Sidhu

Lab Section #3

Lab 4 Report

David Quintanilla

Table of Contents

- Introduction	……………………………………………………………………………3

- Part 1:  SRAM Design ………………………………………………………………4-11

  - Ram Design   ……………………………………………………………….. 4-6

  - Mem_FSM Design ………………………………………………………….. 6-9

  - Top Level Design …………………………………………………………… 9-11

- Part 2: Microprocessor Data Path Design ………………………………………….. 11-13

  - ALU Design ………………………………………………………………… 11-13

- Part 3: Microprocessor Control Path Design…………………………………………13-25

  - FSM ………………………………………………………………………….13-17

    - Data Path Design ……………………………………………………17- 19

    - MUX2_1 Design …………………………………………………… 19- 21

    - MUX4_1 Design …………………………………………………… 21-22

    - DFLIP_A Design ……………………………………………………22- 24

- Part 4: Final 4-bit Microprocessor Design …………………………………………..25-27

  - Final Top Level Design ……………………………………………………..25-27

- Conclusion …………………………………………………………………………. 28

Introduction

Lab 4 introduced the concept of designing a simplified microprocessor. Consisting of several components the microprocessor introduced topics such the SRAM Design and ALU and revisited topics such as FSM’s and top level designs for projects. In Part 1, I wrote a 4 bit number into the 32 address locations of the SRAM. After I finished writing the data I had to read the data from the RAM design. Following the table given in the lab instructions, Part 1 was fairly simple to complete. Part 2, dealt with creating the data path design for the microprocessor. This included creating two different types of MUX’s as well as creating D-Flipflops. The data path in part 2 was very complicated and required several hours of debugging. Using Table 4-1 in the lab instructions I created an ALU that implemented the data path circuit. Part 3 revisited FSM and ask to implement the control path for the microprocessor design.

Lastly, Part 4 was the final top-level design for the microprocessor. Using the formula M0 + (not M1) + Cin to check our results. This lab took longer than most labs, however with several hours of debugging this lab taught me how to properly design a microprocessor.

Part 1: SRAM Design

Ram Design (Verilog)

module ram ( address, data, cs, we, oe );

input cs, we, oe ;

input [3:0] address ;

inout [7:0] data ;

reg [7:0] data_out ;

reg [7:0] mem [0:1023]; //10-bit address bus, 8-bit data bus.

assign data = (cs && oe && !we) ? data_out : 8'bzzzzzzzz;

always @ (cs or we or data or address)

if ( cs && we )

mem[address] = data;

always @ (cs or we or data or address or data)

begin

if (cs && !we && oe)

data_out = mem[address];

end

endmodule

Ram Design(Test Bench)

module ram_tb();

reg cs, we, oe;

reg [3:0] address;

wire [7:0] data;

reg [7:0] data_out ;

ram uut (.cs(cs) , .we(we), .oe(oe) , .address(address) ,.data(data) );

assign data = (cs && oe && !we) ?  8'bzzzzzzzz : data_out;

initial

begin

#10 we = 1'b1; cs = 1'b1; oe = 1'b0; address = 4'b1010; data_out = 8'b1100011;

#10 we = 1'b1; cs = 1'b1; oe = 1'b0; address = 4'b1111; data_out = 8'b1000011;

#10 we = 1'b0; cs = 1'b1; oe = 1'b1; address = 4'b1010; data_out = 8'b1101011;

#10 we = 1'b0; cs = 1'b1; oe = 1'b1; address = 4'b1111; data_out = 8'b1100111;

#10 $stop;

end

initial $monitor($time, ". we = %d, oe = %d, we = %d, address = %d, data_out = %d", we,cs,oe,address,data_out);

endmodule

RAM Wave Form

MEM_FSM (Verilog)

module mem_fsm ( clk, reset, address, data, cs, we, oe );

input clk, reset;

output [5:0] address;

inout [7:0] data;

output cs, we, oe;

reg cs, we, oe;

reg [5:0] address;

reg [7:0] data_reg;

reg [2:0] state;

parameter idle = 3'b000, s1= 3'b001, s2 = 3'b010, s3 = 3'b100;

assign data = data_reg;

always@(posedge clk or posedge reset)

begin

if (reset)

begin

state <= idle;

address <= 0;

end

else

case (state)

idle: begin

state <= s1;

address <= 0;

end

s1: begin

state <= s2;

address <= 0;

end

s2: begin

address <= address + 1;

if(address == 32)

state <= s3;

else

state <= s2;

end

s3:begin

if (address == 0)

state <= s1;

else

begin

address <= address -1;

state <=s3;

end

end

default: begin

state <= idle;

address <= 0;

end

endcase

end

always@(state)

begin

case (state)

idle: begin

cs = 0;

we = 0;

oe = 0;

data_reg = 8'bZZZZZZZZ;

end

s1: begin //writing

cs = 1;

we = 1;

oe = 0;

data_reg = 8'b11000011;

end

s2: begin //writing

cs = 1;

we = 1;

oe = 0;

data_reg = 8'b11000011;

end

s3: begin //reading

cs = 1;

we = 0;

oe = 1;

end

default: begin

cs = 0;

we = 0;

oe = 0;

data_reg = 8'bzzzzzzzz;

end

endcase

end

endmodule

Top (Verilog)

module top(clk, reset, address, data);

input clk, reset;

output [5:0] address;

output [3:0] data;

mem_fsm g1(.clk(clk),.address(address),.data(data),.cs(cs),.we(we),.oe(oe));

ram     g2(.address(address),.data(data),.cs(cs),.we(we),.oe(oe));

endmodule

Top (Test Bench)

module top_tb();

reg clk, reset;

wire  [5:0] address;

wire  [3:0] data;

top uut(.clk(clk), .reset(reset), .address(address), .data(data));

always

begin

#5 clk = ~clk;

end

initial

begin

clk = 0;

#10 reset = 1;

#10 reset = 0;

#700 $stop;

end

endmodule

Top (Wave Form)

Part 2: Microprocessor Data Path Design

ALU Design (Verilog)

module ALU(A, B, S, cin, y);

input  A, B, cin;

input  [2:0] S;

output [3:0] y;

reg    [3:0] y;

always@( S or A or B or cin )

begin

case (S)

0 : y = A + B + cin;

1 : y = A + ~B + cin;

2 : y = B;

3 : y = A;

4 : y = (A & B);

5 : y = (A | B);

6 : y = ~A;

7 : y = (A ^ B);

default : y = 0;

endcase

end

endmodule

ALU Design (Test Bench)

module ALU_tb;

reg[3:0] A, B;

reg[2:0] S;

reg cin;

wire[3:0] y;

ALU uut(A, B, S, cin, y);

initial begin

S = 0; A = 1; B = 2; cin = 1;

#10 S = 1; A = 1; B = 0; cin = 1;

#10 S = 2; A = 1; B = 0; cin = 1;

#10 S = 3; A = 1; B = 0; cin = 1;

#10 S = 4; A = 1; B = 0; cin = 1;

#10 S = 6; A = 1; B = 0; cin = 1;

#10 S = 7; A = 1; B = 0; cin = 1;

#10 $stop;

end

endmodule

ALU Design (Wave Form)

Part 3 Microprocessor Control Path Design

FSM (Verilog)

module FSM(clk, reset,  clr, w, ce, sel, s);

input clk, reset;

output reg  clr;

output reg [2:0] w, s;

output reg [1:0] sel;

output reg [3:0] ce;

reg [2:0] cs, ns;

parameter s0=0, s1=1, s2=2, s3=3, s4=4, s5=5;

always@(posedge clk or posedge reset) begin

if( reset ) begin

cs <= s0;

end else begin

cs <= ns;

end

end

always@(cs) begin

case(cs)

s0 : begin

clr = 1'b1;

w = 3'b100;

s = 3'b010;

sel = 2'b00;

ce = 4'b0000;

ns <= s1;

end

s1 : begin

clr = 1'b0;

w = 3'b100;

s = 3'b010;

sel = 2'b00;

ce = 4'b1111;

ns <= s2;

end

s2 : begin

clr = 1'b0;

w = 3'b100;

s = 3'b010;

sel = 2'b00;

ce = 4'b1111;

ns <= s3;

end

s3 : begin

clr = 1'b0;

w = 3'b100;

s = 3'b001;

sel = 2'b01;

ce = 4'b1111;

ns <= s4;

end

s4 : begin

clr = 1'b0;

w = 3'b100;

s = 3'b001;

sel = 2'b01;

ce = 4'b0111;

ns <= s5;

end

s5 : begin

clr = 1'b0;

w = 3'b100;

s = 3'b001;

sel = 2'b01;

ce = 4'b0111;

ns <= s5;

end

//default : ns = s0;

endcase

end

endmodule

FSM (Test Bench)

module FSM_tb();

reg clk, reset;

wire clr;

wire [1:0] sel;

wire [2:0] w, s;

wire [3:0] ce;

FSM uut(.clk(clk), .reset(reset), .clr(clr), .ce(ce), .w(w), .s(s), .sel(sel));

always

begin

#5 clk = ~clk;

end

initial

begin

clk = 0;

#2 reset = 1;

#10 reset = 0;

#150

$stop;

end

endmodule

FSM (Wave Form)

Data Path (Verilog)

module MUX_D_Combo(m0, m1, m2, cin, clr, w, ce, sel, s, clk, r0, r1, r2);

input clk, clr, cin;

input[3:0] m0,m1,m2,ce;

input[2:0] w, s;

input[1:0] sel;

output[3:0] r0, r1, r2;

wire [3:0] y0, y1, y2, y3, a, b;

MUX2_1 g1(.d0(m0),.d1(a), .s(w[0]), .y(y0));

MUX2_1 g2(.d0(m1),.d1(a), .s(w[1]), .y(y1));

MUX2_1 g3(.d0(m2),.d1(a), .s(w[2]), .y(y2));

DFLIP_A d1(.clk(clk), .clr(clr), .ce(ce[0]),.d(y0),.q(r0));

DFLIP_A d2(.clk(clk), .clr(clr), .ce(ce[1]),.d(y1),.q(r1));

DFLIP_A d3(.clk(clk), .clr(clr), .ce(ce[2]),.d(y2),.q(r2));

DFLIP_A d4(.clk(clk), .clr(clr), .ce(ce[3]),.d(y3),.q(a));

MUX4_1 g4(.d0(r0),.d1(r1),.d2(r2),.d3(4'b0000),.sel(sel),.y(b));

ALU a1(.a(a),.b(b),.s(s),.cin(cin),.y(y3));

Endmodule

Data Path (Test Bench)

`timescale 1ns / 1ps

module MUX_D_Combo_tb;

reg[3:0] m0, m1, m2, ce;

reg[2:0] w, s;

reg[1:0] sel;

reg clk, clr, cin;

wire [3:0] r0, r1, r2;

MUX_D_Combo uut(.m0(m0), .m1(m1), .m2(m2), .cin(cin), .clr(clr), .w(w), .ce(ce), .sel(sel), .s(s), .clk(clk), .r0(r0), .r1(r1), .r2(r2));

initial begin

clr = 1'b1; w = 3'b000; ce = 4'b0000; sel = 2'b00; s = 3'b000; clk = 1'b0; m2 = 4'b0000;

m0 = 4'b0101; m1 = 4'b0011;

cin = 1;

end

always #1 clk = ~clk;

initial begin

#10; clr = 1'b0;

#10; ce = 4'b0011; sel = 2'b00; s = 3'b010;

#10; ce = 4'b1011;

#10; ce = 4'b0011; sel = 2'b01; s = 3'b001;

#10; ce = 4'b1000;

#10; ce = 4'b0100; w = 3'b100;

#10; w = 3'b000; ce = 4'b0000; sel = 2'b00; s = 3'b000;

#10; $stop;

end

endmodule

Data Path (Wave Form)

MUX2_1 (Verilog)

module MUX2_1(d0, d1, s, y);

input [3:0]	 d1, d0;

input s;

output reg [3:0] y;

always@( s or d1 or d0 )

begin

if (s)

y = d1;

else

y = d0;

end

endmodule

MUX2_1 (Test Bench)

module MUX2_1_tb();

reg[3:0] d0, d1;

reg s;

wire[3:0] y;

MUX2_1 u1(d0, d1, s, y);

initial begin

d1 = 0; d0 = 0; s = 0;

#10 d1 = 0; d0 = 0; s = 1;

#10 d1 = 0; d0 = 1; s = 0;

#10 d1 = 0; d0 = 1; s = 1;

#10 d1 = 1; d0 = 0; s = 0;

#10 d1 = 1; d0 = 0; s = 1;

#10 d1 = 1; d0 = 1; s = 0;

#10 d1 = 1; d0 = 1; s = 1;

#10 $stop;

end

endmodule

MUX2_1 (Wave Form)

MUX4_1 (Verilog)

module MUX4_1(d0, d1, d2, d3, sel, y);

input [3:0] d3, d2, d1, d0;

input [1:0] sel;

output reg  [3:0] y;

always@( sel or d3 or d2 or d1 or d0 )

begin

case (sel)

0 : y = d0;

1 : y = d1;

2 : y = d2;

3 : y = d3;

endcase

end

endmodule

MUX4_1 (Test Bench)

module MUX4_1_tb;

reg[3:0] d0, d1, d2, d3;

reg[1:0] sel;

wire[3:0] y;

MUX4_1 u1(.d0(d0), .d1(d1), .d2(d2), .d3(d3), .sel(sel), .y(y));

initial

begin

d0 = 0; d1 = 1; d2 = 2; d3 = 3; sel = 2'b00;

#10 d0 = 0; d1 = 1; d2 = 2; d3 = 3; sel = 2'b01;

#10 d0 = 0; d1 = 1; d2 = 2; d3 = 3; sel = 2'b10;

#10 d0 = 0; d1 = 1; d2 = 2; d3 = 3; sel = 2'b11;

#10 $stop;

end

endmodule

MUX4_1 (Wave Form)

DFLIP_A (Verilog)

module DFLIP_A (clk, clr, d, ce, q);

input[3:0] d;

input clk, ce, clr;

output reg[3:0] q;

always@(posedge clr or posedge clk)

begin

if(clr) q <= 0;

else if(ce) q <= d;

end

endmodule

DFLIP_A (Test Bench)

module DFLIP_A_tb();

reg[3:0] d;

reg clk, clr, ce;

wire[3:0] q;

DFLIP_A utt(.clk(clk), .clr(clr), .d(d), .q(q), .ce(ce));

initial

clk = 0;

always

#10 clk = ~clk;

initial begin

clr = 1; d = 1; ce = 1;

#20 clr = 0;

#20 ce = 0;

#40

$stop;

end

endmodule

DFLIP_A (Wave Form)

Part 4: Final 4-bit Microprocessor Design

Top Final (Verilog)

module Top_Final( clk, reset, m0, m1, m2, cin, r0, r1, r2);

input clk, cin, reset;

input [3:0] m0, m1, m2;

output [3:0] r0, r1, r2;

wire [2:0] w, s;

wire [1:0] sel;

wire [3:0] ce;

wire clr;

FSM fsm1(.clk(clk), .reset(reset),  .clr(clr), .w(w), .ce(ce), .sel(sel), .s(s));

MUX_D_Combo mux_d_combo1(.clk(clk), .clr(clr), .w(w), .ce(ce), .sel(sel), .s(s), .m0(m0), .m1(m1), .m2(m2), .cin(cin), .r0(r0), .r1(r1), .r2(r2));

Endmodule

Top Final (Test Bench)

module Top_Final_tb();

reg clk, reset, cin;

reg [3:0] m0,m1,m2;

wire [3:0] r0,r1,r2;

Top_Final uut(.clk(clk), .reset(reset), .cin(cin), .m0(m0), .m1(m1), .m2(m2), .r0(r0), .r1(r1), .r2(r2));

always begin

#5 clk = ~clk;

end

initial begin

clk = 0; reset = 1; cin = 1; m0=0; m1=0; m2=0;

//#10 reset = 0;

// m0=4'b1000;

//m1=4'b0001;

//m2=4'b1111;

//cin = 1;

#10 m0 = 3; reset = 0;

m1 = 13;

//m2 = 2;

#150 $stop;

end

endmodule

Top Final (Wave Form)

Conclusion

To conclude the main objective of this lab; which was to design a simple microprocessor that implemented the logic equation R2 = M0 +not (M1) + Cin was successfully created. The use of hierarchal design within the different modules was again used in order to link all modules together and create instances. It was again great practice for designing combinational and sequential circuits. I believe that this lab explored several topics that I was interested in on retouching and furthered my understanding of advance logic designs.